QUESTION 1

What is the relationship between the digital signal and the presence of the carrier in the ask signal?

When the digital signal is high the carrier signal is present at the output. When digital signal is low the carrier signal is absent. The digital signal acts as a control for the Dual Analog Switch, "keying" the carrier on and off. This is why the process is called Amplitude Shift Keying (ASK)

QUESTION 2

What is the ask signal's voltage when the digital signal is logic-0? tip: if you're not sure, briefly set the channel 2 input coupling control to the GND position.

The ASK signal's voltage is 0V (zero volts) when the digital signal is logic-0.

QUESTION 3

what feature of the ASK signal suggest that its an am signal? tip: if youre not sure, see the preliminary discussion.

The feature that suggests it is an AM (Amplitude Modulation) signal is that the amplitude of the carrier wave is being changed (modulated) by the digital message signal.

QUESTION 4

Why is the recovered digital signal not a perfect copy of the original? ​

The recovered signal is not perfect because of the limitations of filtering and ripple. The Low-pass Filter is designed to remove the high-frequency 100 kHz carrier, but it cannot perfectly recreate the "instantaneous" vertical edges of a digital square wave.

QUESTION 5

What can be used to "clean-up" the recovered digital signal?

​A Comparator (or a Schmitt Trigger) can be used to "clean up" the signal

QUESTION 6

How does the comparator turn the slow rising voltages of the recovered digital signal into sharp transitions?

The comparator turns slow-rising voltages into sharp transitions by comparing the input signal to a fixed reference voltage (the threshold). Because the comparator has extremely high gain, its output can only exist in two states: ​High, Whenever the input signal rises even slightly above the reference voltage. ​Low, Whenever the input signal falls even slightly below the reference voltage.